MINI-PROJECT

RTL CODE:

`timescale 1ns / 1ps

module uart_top #(

parameter CLKS_PER_BIT = 10417,

parameter DB_CYCLES    = 1_000_000

)(

input         clk,

input  [15:0] sw,

input  [3:0]  btn,

input         UART_rxd,

output        UART_txd,

output [15:0] led

);

localparam IDLE = 2'd0, START = 2'd1, DATA = 2'd2, STOP = 2'd3;

reg rst_s1 = 0, rst = 0;

reg b1 = 0, b2 = 0, b_stable = 0, send_pulse = 0;

reg [20:0] db_cnt = 0;

always @(posedge clk) begin

rst_s1 <= btn[3]; rst <= rst_s1;

b1 <= btn[0]; b2 <= b1; send_pulse <= 1'b0;

if (b2 == b_stable) db_cnt <= 0;

else if (db_cnt == DB_CYCLES-1) begin

b_stable <= b2; db_cnt <= 0;

if (b2) send_pulse <= 1'b1;

end else db_cnt <= db_cnt + 1'b1;

end

reg [1:0]  tx_state = IDLE;

reg [15:0] tx_cnt   = 0;

reg [2:0]  tx_idx   = 0;

reg [7:0]  tx_sh    = 0;

reg        tx       = 1'b1;

wire       tx_busy  = (tx_state != IDLE);

always @(posedge clk) begin

if (rst) begin

tx_state <= IDLE; tx <= 1'b1; tx_cnt <= 0; tx_idx <= 0;

end else case (tx_state)

IDLE: begin

tx <= 1'b1; tx_cnt <= 0; tx_idx <= 0;

if (send_pulse) begin tx_sh <= sw[7:0]; tx_state <= START; end

end

START: begin

tx <= 1'b0;

if (tx_cnt == CLKS_PER_BIT-1) begin tx_cnt <= 0; tx_state <= DATA; end

else tx_cnt <= tx_cnt + 1'b1;

end

DATA: begin

tx <= tx_sh[tx_idx];

if (tx_cnt == CLKS_PER_BIT-1) begin

tx_cnt <= 0;

if (tx_idx == 3'd7) begin tx_idx <= 0; tx_state <= STOP; end

else tx_idx <= tx_idx + 1'b1;

end else tx_cnt <= tx_cnt + 1'b1;

end

STOP: begin

tx <= 1'b1;

if (tx_cnt == CLKS_PER_BIT-1) begin tx_cnt <= 0; tx_state <= IDLE; end

else tx_cnt <= tx_cnt + 1'b1;

end

endcase

end

wire rx_line = sw[15] ? tx : UART_rxd;

reg  rx_s1 = 1, rx_s2 = 1;

reg [1:0]  rx_state = IDLE;

reg [15:0] rx_cnt   = 0;

reg [2:0]  rx_idx   = 0;

reg [7:0]  rx_sh    = 0;

reg [7:0]  rx_data  = 0;

reg        rx_toggle = 0, frame_err = 0;

always @(posedge clk) begin

rx_s1 <= rx_line; rx_s2 <= rx_s1;

if (rst) begin

rx_state <= IDLE; rx_cnt <= 0; rx_idx <= 0;

rx_data <= 0; rx_toggle <= 0; frame_err <= 0;

end else case (rx_state)

IDLE: begin

rx_cnt <= 0; rx_idx <= 0;

if (rx_s2 == 1'b0) rx_state <= START;

end

START: begin

if (rx_cnt == (CLKS_PER_BIT-1)/2) begin

rx_cnt <= 0;

rx_state <= (rx_s2 == 1'b0) ? DATA : IDLE;

end else rx_cnt <= rx_cnt + 1'b1;

end

DATA: begin

if (rx_cnt == CLKS_PER_BIT-1) begin

rx_cnt <= 0;

rx_sh[rx_idx] <= rx_s2;

if (rx_idx == 3'd7) begin rx_idx <= 0; rx_state <= STOP; end

else rx_idx <= rx_idx + 1'b1;

end else rx_cnt <= rx_cnt + 1'b1;

end

STOP: begin

if (rx_cnt == CLKS_PER_BIT-1) begin

rx_cnt <= 0; rx_state <= IDLE;

frame_err <= ~rx_s2;

if (rx_s2) begin rx_data <= rx_sh; rx_toggle <= ~rx_toggle; end

end else rx_cnt <= rx_cnt + 1'b1;

end

endcase

end

assign UART_txd = tx;

assign led = {5'b00000, rx_toggle, frame_err, tx_busy, rx_data};

endmodule

TESTBENCH:

`timescale 1ns / 1ps

module tb_uart;

localparam CPB    = 16;                 // small divider for simulation

localparam BIT_T  = CPB * 10;           // bit time in ns (10 ns clock)

reg         clk = 0;

reg  [15:0] sw  = 0;

reg  [3:0]  btn = 0;

reg         rxd = 1;

wire        txd;

wire [15:0] led;

uart_top #(.CLKS_PER_BIT(CPB), .DB_CYCLES(4)) dut (

.clk(clk), .sw(sw), .btn(btn),

.UART_rxd(rxd), .UART_txd(txd), .led(led));

always #5 clk = ~clk;

integer errors = 0;

task press_send;

begin

btn[0] = 1; repeat (20) @(posedge clk);

btn[0] = 0; repeat (20) @(posedge clk);

end

endtask

// act like the PC: drive a byte serially into UART_rxd

task send_serial(input [7:0] b);

integer i;

begin

rxd = 0; #(BIT_T);

for (i = 0; i < 8; i = i + 1) begin rxd = b[i]; #(BIT_T); end

rxd = 1; #(BIT_T);

end

endtask

task check_led(input [7:0] exp);

begin

if (led[7:0] !== exp) begin

$display("FAIL: expected %h, got %h", exp, led[7:0]);

errors = errors + 1;

end else

$display("PASS: received %h", led[7:0]);

end

endtask

// independent monitor on the TX line: decodes what the DUT transmits

reg [7:0] mon;

integer   k;

always @(negedge txd) begin

#(BIT_T/2);                              // middle of start bit

for (k = 0; k < 8; k = k + 1) begin #(BIT_T); mon[k] = txd; end

#(BIT_T);                                // middle of stop bit

if (txd !== 1'b1) begin $display("FAIL: stop bit not 1"); errors = errors + 1; end

$display("[%0t ns] TX monitor decoded %h", $time, mon);

end

initial begin

// reset

btn[3] = 1; repeat (10) @(posedge clk);

btn[3] = 0; repeat (10) @(posedge clk);

// Test 1: internal loopback (sw15 = 1)

sw[15] = 1;

sw[7:0] = 8'hA5; press_send; #(BIT_T*12); check_led(8'hA5);

sw[7:0] = 8'h3C; press_send; #(BIT_T*12); check_led(8'h3C);

sw[7:0] = 8'h00; press_send; #(BIT_T*12); check_led(8'h00);

sw[7:0] = 8'hFF; press_send; #(BIT_T*12); check_led(8'hFF);

// Test 2: external pin / PC mode (sw15 = 0)

sw[15] = 0;

send_serial(8'h41); #(BIT_T*2); check_led(8'h41);   // 'A'

send_serial(8'h7E); #(BIT_T*2); check_led(8'h7E);

if (errors == 0) $display("ALL TESTS PASSED");

else             $display("%0d TEST(S) FAILED", errors);

$finish;

end

endmodule

CONSTRAINT FILE:

## Boolean board (Spartan-7 XC7S50) constraints for uart_top

## 100 MHz clock

set_property -dict {PACKAGE_PIN F14 IOSTANDARD LVCMOS33} [get_ports {clk}]

create_clock -period 10.000 -name sys_clk [get_ports {clk}]

## Bank 0 voltage

set_property CFGBVS VCCO [current_design]

set_property CONFIG_VOLTAGE 3.3 [current_design]

## Slide switches

set_property -dict {PACKAGE_PIN V2 IOSTANDARD LVCMOS33} [get_ports {sw[0]}]

set_property -dict {PACKAGE_PIN U2 IOSTANDARD LVCMOS33} [get_ports {sw[1]}]

set_property -dict {PACKAGE_PIN U1 IOSTANDARD LVCMOS33} [get_ports {sw[2]}]

set_property -dict {PACKAGE_PIN T2 IOSTANDARD LVCMOS33} [get_ports {sw[3]}]

set_property -dict {PACKAGE_PIN T1 IOSTANDARD LVCMOS33} [get_ports {sw[4]}]

set_property -dict {PACKAGE_PIN R2 IOSTANDARD LVCMOS33} [get_ports {sw[5]}]

set_property -dict {PACKAGE_PIN R1 IOSTANDARD LVCMOS33} [get_ports {sw[6]}]

set_property -dict {PACKAGE_PIN P2 IOSTANDARD LVCMOS33} [get_ports {sw[7]}]

set_property -dict {PACKAGE_PIN P1 IOSTANDARD LVCMOS33} [get_ports {sw[8]}]

set_property -dict {PACKAGE_PIN N2 IOSTANDARD LVCMOS33} [get_ports {sw[9]}]

set_property -dict {PACKAGE_PIN N1 IOSTANDARD LVCMOS33} [get_ports {sw[10]}]

set_property -dict {PACKAGE_PIN M2 IOSTANDARD LVCMOS33} [get_ports {sw[11]}]

set_property -dict {PACKAGE_PIN M1 IOSTANDARD LVCMOS33} [get_ports {sw[12]}]

set_property -dict {PACKAGE_PIN L1 IOSTANDARD LVCMOS33} [get_ports {sw[13]}]

set_property -dict {PACKAGE_PIN K2 IOSTANDARD LVCMOS33} [get_ports {sw[14]}]

set_property -dict {PACKAGE_PIN K1 IOSTANDARD LVCMOS33} [get_ports {sw[15]}]

## LEDs

set_property -dict {PACKAGE_PIN G1 IOSTANDARD LVCMOS33} [get_ports {led[0]}]

set_property -dict {PACKAGE_PIN G2 IOSTANDARD LVCMOS33} [get_ports {led[1]}]

set_property -dict {PACKAGE_PIN F1 IOSTANDARD LVCMOS33} [get_ports {led[2]}]

set_property -dict {PACKAGE_PIN F2 IOSTANDARD LVCMOS33} [get_ports {led[3]}]

set_property -dict {PACKAGE_PIN E1 IOSTANDARD LVCMOS33} [get_ports {led[4]}]

set_property -dict {PACKAGE_PIN E2 IOSTANDARD LVCMOS33} [get_ports {led[5]}]

set_property -dict {PACKAGE_PIN E3 IOSTANDARD LVCMOS33} [get_ports {led[6]}]

set_property -dict {PACKAGE_PIN E5 IOSTANDARD LVCMOS33} [get_ports {led[7]}]

set_property -dict {PACKAGE_PIN E6 IOSTANDARD LVCMOS33} [get_ports {led[8]}]

set_property -dict {PACKAGE_PIN C3 IOSTANDARD LVCMOS33} [get_ports {led[9]}]

set_property -dict {PACKAGE_PIN B2 IOSTANDARD LVCMOS33} [get_ports {led[10]}]

set_property -dict {PACKAGE_PIN A2 IOSTANDARD LVCMOS33} [get_ports {led[11]}]

set_property -dict {PACKAGE_PIN B3 IOSTANDARD LVCMOS33} [get_ports {led[12]}]

set_property -dict {PACKAGE_PIN A3 IOSTANDARD LVCMOS33} [get_ports {led[13]}]

set_property -dict {PACKAGE_PIN B4 IOSTANDARD LVCMOS33} [get_ports {led[14]}]

set_property -dict {PACKAGE_PIN A4 IOSTANDARD LVCMOS33} [get_ports {led[15]}]

## Push buttons

set_property -dict {PACKAGE_PIN J2 IOSTANDARD LVCMOS33} [get_ports {btn[0]}]

set_property -dict {PACKAGE_PIN J5 IOSTANDARD LVCMOS33} [get_ports {btn[1]}]

set_property -dict {PACKAGE_PIN H2 IOSTANDARD LVCMOS33} [get_ports {btn[2]}]

set_property -dict {PACKAGE_PIN J1 IOSTANDARD LVCMOS33} [get_ports {btn[3]}]

## UART (USB "PROG UART" port)

set_property -dict {PACKAGE_PIN V12 IOSTANDARD LVCMOS33} [get_ports {UART_rxd}]

set_property -dict {PACKAGE_PIN U11 IOSTANDARD LVCMOS33} [get_ports {UART_txd}]

SIMULATION WAVEFORM:

SCHEMATIC:

IMPLEMENTED DESGIN:

OUTPUT ON TERMINAL: